Lyophilized Product Harvest and Transfer-Out Isolator

Process Introduction Video — English Voice-Over Script

Version dated 13 September 2026   |   Translated from the Chinese voice-over script

1.  Voice-Over Script

Timecodes are cue-in points; narration is written in a third-person descriptive register suited to a customer-facing process video.

| Timecode | English Voice-Over |
| --- | --- |
| 00:20 | A continuous liner of the required length is prepared and fitted onto the bag-out port of the isolator. |
| 00:50 | The lyophilized product is discharged from the membrane lyophilization tray into a pharmaceutical-grade low-density polyethylene bag. Where necessary, a stainless steel scoop is used to assist product recovery. |
| 01:20 | The bag is closed and secured with a zip tie. |
| 01:26 | The outer surface of the polyethylene bag is wiped down. |
| 01:39 | The sealed bag is passed into the continuous liner mounted on the bag-out port. |
| 01:50 | The required length of continuous liner is drawn out. |
| 02:03 | The continuous liner is crimp-sealed and severed using a Dover CrimpLoc tool. |
| 02:54 | The bagged product is placed into an aluminum foil bag. |
| 03:00 | Desiccant is added as required. |
| 03:12 | The foil bag is heat-sealed. |
| 03:34 | The sealed package is placed into an HDPE drum. |
| 03:49 | [Production note — not voiced] End of voice-over; remaining footage to be trimmed. |

2.  Bilingual Reference (for internal review)

Provided so that process owners can verify the English against the approved Chinese wording line by line.

| Timecode | Chinese Source | English Voice-Over |
| --- | --- | --- |
| 00:20 | 按需准备一定长度的连续性塑料袋，安装在隔离器的袋出筒上 | A continuous liner of the required length is prepared and fitted onto the bag-out port of the isolator. |
| 00:50 | 将冻干膜盒中的产品倒入药用级低密度聚乙烯袋中，必要时可以使用不锈钢铲子辅助收料 | The lyophilized product is discharged from the membrane lyophilization tray into a pharmaceutical-grade low-density polyethylene bag. Where necessary, a stainless steel scoop is used to assist product recovery. |
| 01:20 | 拉扣封口 | The bag is closed and secured with a zip tie. |
| 01:26 | 清洁PE袋外表面 | The outer surface of the polyethylene bag is wiped down. |
| 01:39 | 将包装袋传入安装在袋出筒的连续性塑料袋中 | The sealed bag is passed into the continuous liner mounted on the bag-out port. |
| 01:50 | 按需量取一定长度的连续性塑料袋 | The required length of continuous liner is drawn out. |
| 02:03 | 用Dover CrimpLoc工具密封剪断连续性塑料袋 | The continuous liner is crimp-sealed and severed using a Dover CrimpLoc tool. |
| 02:54 | 将包装袋放入铝箔袋中 | The bagged product is placed into an aluminum foil bag. |
| 03:00 | 按需放入干燥剂 | Desiccant is added as required. |
| 03:12 | 热封 | The foil bag is heat-sealed. |
| 03:34 | 放入HDPE桶中 | The sealed package is placed into an HDPE drum. |
| 03:49 | end，后面的视频剪掉即可 | [Production note — not voiced] End of voice-over; remaining footage to be trimmed. |

3.  Terminology Decisions

Key equipment and packaging terms rendered with the containment vocabulary used by pharmaceutical customers.

| Chinese Term | English Term Used | Basis / Point to Confirm |
| --- | --- | --- |
| 冻干收料传出隔离器 | Lyophilized product harvest and transfer-out isolator | Containment enclosure for discharging bulk lyophilized product and transferring it out. |
| 连续性塑料袋 | Continuous liner | Industry-standard containment term for continuous tubular liner film. |
| 袋出筒 | Bag-out port | The liner-mounting port on the isolator, also referred to as a continuous-liner port. |
| 冻干膜盒 | Membrane lyophilization tray | Membrane-covered bulk lyophilization tray; confirm against the actual tray brand shown on screen. |
| 药用级低密度聚乙烯袋 | Pharmaceutical-grade low-density polyethylene (LDPE) bag | First mention spells out LDPE; later mentions use “polyethylene bag”. |
| 拉扣封口 | Closed and secured with a zip tie | Confirm the closure device shown on screen (zip tie vs. zip-lock closure). |
| 密封剪断 | Crimp-sealed and severed | Matches Dover CrimpLoc tool terminology. |
| 热封 | Heat-sealed | Applies to the aluminum foil outer bag. |